Local vs. Global Localization & Trajectory Precision

Analysis of top-team practice and a concrete 2027 plan for MechaRAMS

Prepared June 23, 2026  ·  Continuation of the FRC vision/localization investigation

Contents

# 1. Executive summary

Yes — the split you intuited is real, and it is exactly how the strongest teams frame the problem. In his championship talk “Beyond the Coprocessor,” 6328’s lead explicitly separates global field localization (good enough to drive a path) from precise local positioning (good enough to align/score). They are different accuracy budgets and they deserve different solutions. Your 2026 robot used one global estimate for both jobs, and the angular slop that is tolerable for driving (2–3°) is fatal for aiming.

On aiming, the answer is nuanced. 6328 — the team behind Northstar — does not use a turret or a target-relative camera. They aim the whole chassis at a fixed field coordinate computed from the global pose, with a velocity “lookahead” for shooting on the move. They make the global estimate good enough that whole-robot aim lands. That works only because their multi-camera fusion is excellent and logged, so they can trust it. Your complaint — “no way to check” — is the missing piece: they verify in replay logs, you had nothing on the turret. A dedicated boresight camera that sees the goal tag is the cheapest way to buy both the angular precision and the independent check.

On driving, the smoking gun is in their code. 6328’s trajectory follower ends on time — literally timer.hasElapsed(trajectory.getTotalTime()) — the exact behavior that frustrated you in PathPlanner. The difference is they never let a time-based path finish a precise move. They interrupt it early on a spatial condition and hand off to a separate DriveToPose controller that runs until the robot is within 1 cm and 1° of the goal, regardless of time. That handoff is the whole trick.

Bottom line:  Keep PathPlanner/Choreo for transit, but stop trusting it to arrive. Add a drive-to-pose end-controller for the final approach, and add a shooter-boresight camera so aiming is measured directly instead of inferred from a global pose. Both are summer-sized projects. Details and code skeletons follow.

# 2. Restating the two problems precisely

Problem 1 — aiming. You localized globally but not precisely enough for the shot. A 2–3° heading error moved the shot off target, and with no camera on the turret you had no independent measurement of where you were actually pointed relative to the goal. The error was invisible until the game piece missed.

Problem 2 — driving. PathPlanner follows a time-parameterized trajectory: it computes that a path should take, say, 2.0 s and it stops driving at t = 2.0 s whether or not you are at the endpoint. For transit that is fine; when the endpoint position is what matters (scoring, pickup, a precise shot pose) it leaves you wherever the clock ran out.

These are two faces of the same root cause: a single, time-and-pose-based control loop being asked to deliver centimeter/degree precision that it was never structured to guarantee.

# 3. Part A — Do teams localize locally and globally separately?

## 3.1 The conceptual split (the framework to adopt)

The cleanest articulation comes from 6328’s “Beyond the Coprocessor” talk, which sorts every localization need into a small number of categories. The two that matter here:

| Category | Accuracy budget | What it drives | Right tool |
| --- | --- | --- | --- |
| Global field localization | ~5–20 cm, a few degrees | Path following, knowing which half of the field you’re in, long-range strategy | Fused multi-camera AprilTag + wheel/gyro odometry (your current system) |
| Precise local positioning | sub-cm / sub-degree, relative to one target | Aligning to score, lining up a shot, pickup | A direct measurement to the target itself (a tag-relative camera, or a very good global pose + verification) |

Your 2026 setup collapsed both rows into the first tool. The insight is that the second row often wants a measurement of the target, not a better estimate of yourself on the field.

## 3.2 What 6328 actually does for aiming (global-pose, no turret)

Reading their 2026 code: aiming is computed entirely from the global estimated pose to a fixed field constant — there is no turret and no camera pointed at the goal. The launch calculator takes the estimated pose, projects it forward by the robot’s velocity × time-of-flight (shoot-on-the-move), and computes the chassis heading that points the launcher at the hub:

// 6328 LaunchCalculator.getDriveAngleWithLauncherOffset (abridged)

Translation2d target = FieldConstants.Hub.topCenterPoint;          // fixed field point

Rotation2d fieldToHubAngle =

target.minus(robotPose.getTranslation()).getAngle();       // angle from GLOBAL pose

// ... plus a correction for the launcher being off the robot center ...

Rotation2d driveAngle = fieldToHubAngle.plus(hubAngle).plus(launcherYaw);

// DriveCommands then servos chassis omega so robot heading -> driveAngle,

// adding velocity "lookahead" so a moving shot still lands.

Read this carefully:  the entire shot depends on robotPose being correct. If the global estimate is off by 3°, the shot is off by 3° — there is no inner loop that looks at the goal and corrects. 6328 survives this because (a) their multi-camera fusion keeps the pose tight, and (b) every shot is logged and replayed so a bad estimate is caught and the fusion is tuned. You had neither, which is why the same architecture failed you.

## 3.3 The alternative: a target-relative (boresight / turret) camera

The other school of thought closes the loop on the target directly. A camera bore-sighted with the shooter (or mounted on a turret) sees the goal AprilTag and reports the bearing to the tag — a pure angular measurement that is immune to global translational drift. You aim until that bearing is zero. This is precisely the “precise local positioning” tool, and it is the direct fix for an angular-error problem.

Community experience (Chief Delphi, 2026 season threads) is candid about the trade:

- For: direct, drift-proof, and it gives you the independent check you said you lacked — you can literally compare “where the pose says the goal is” against “where the camera sees it.”

- Against: the tag must stay in the camera’s view through the whole maneuver. Multiple teams reported alignment “freezing” when the robot drove close and the tag left the frame.

- Common compromise: most teams that tried pure tag-relative servoing in 2025 fell back to MegaTag2-pose + PID because it was more robust match-to-match — but they were aligning to a reef/scoring face at close range, not aiming a long shot where angular precision dominates. Your problem is the one case where a boresight camera earns its keep.

## 3.4 How five teams actually aim (cross-team comparison)

Looking beyond 6328, the same two-tier idea recurs — but how much of the “local” half a team builds depends mostly on whether they have a turret.

| Team (year) | Aims using | Mechanism | Local/global split? |
| --- | --- | --- | --- |
| 6328 (2026) | Global pose → fixed hub point | Chassis heading + velocity lookahead | Global only; verified in replay logs |
| 1768 Nashoba (2026) | Global pose (ShootingUtil) | Chassis heading snap; atShootingSetpoint gate before wheel-lock | Global only |
| 3467 Windham (2026) | Global pose | Chassis aim / DriveToPose; vision cameras used for object detection | Global only |
| 2910 Jack in the Bot (2024) | Measured tag angle when visible, else global pose | TURRET: turretAngle + measuredAprilTagAngle, falls back to atan2 from pose | Yes — true local + global hybrid |
| 1678 Citrus (2024) | Separate vision GoalTracker + global pose estimator | Chassis snapHeading; dedicated goal tracker is the local signal | Yes — separate local tracker |

Pattern:  turret teams (2910) get the cleanest split because the turret keeps the goal in frame while the chassis does other things; chassis-aim teams either rely entirely on the global pose (6328, 1768, 3467) or bolt on a separate goal tracker (1678). In 2026 you had only the global half and no tracker — that is exactly the gap a shooter-boresight camera fills.

## 3.5 Verdict for MechaRAMS

Recommendation:  Run a hybrid, with the global pose as the coarse aim and a shooter-boresight camera as the fine correction and the check.

- Primary (coarse): keep global-pose chassis aim with velocity lookahead, like 6328. It is what enables shoot-on-the-move and it points you roughly right even when the goal tag is briefly occluded.

- Fine correction: a global-shutter camera bore-sighted to the launcher. When it sees the goal tag, PID the last degree or two of chassis (or turret) heading on the measured bearing, not on the pose. This is the layer that kills the 2–3° error.

- The check you were missing: continuously log poseDerivedBearing − cameraMeasuredBearing. A persistent gap is a calibration/extrinsics error you can now see — in the pit, not by watching game pieces miss.

- Don’t skip the cheap win: the global-pose fixes from the prior analysis (MegaTag2 yaw fused with infinite std-dev, fuse all cameras, distance² weighting) shrink the 2–3° at the source, so the boresight loop has less work to do.

# 4. Part B — Driving precision: beyond time-based path following

## 4.1 Why this happens (and that it isn’t unique to PathPlanner)

It is tempting to blame PathPlanner, but every mainstream trajectory follower is time-parameterized — including Choreo and 6328’s own. Here is the literal end condition from 6328’s DriveTrajectory:

// 6328 DriveTrajectory.java

@Override

public boolean isFinished() {

return timer.hasElapsed(trajectory.getTotalTime());   // ends on the CLOCK

}

// inside execute(): PID on position error + feedforward from the sample

double xOutput = xController.calculate(pose.getX(), sample.x) + sample.vx;

double yOutput = yController.calculate(pose.getY(), sample.y) + sample.vy;

Two things to take from this. First, the follower does correct toward the path with PID — it is not open-loop — but it terminates on time, so any residual error at the final sample is simply left uncorrected. Second, PathPlanner’s PPHolonomicDriveController is the same shape: feedforward from the time-sampled state plus PID on the error. Switching to Choreo would give you nicer, dynamics-aware trajectories but the same time-based ending. The fix is not a better follower; it is a different final controller.

## 4.2 The fix top teams use: trajectory for transit, drive-to-pose for arrival

6328 builds precise autos by running the trajectory, interrupting it early on a spatial condition, then handing the last stretch to DriveToPose, which ends on a position tolerance. Their actual auto code:

// 6328 AutoCommands (abridged) - the handoff pattern

new DriveTrajectory(trajectory, drive)

.until(() -> xCrossed(handoffLine, true))   // bail out of the timed path EARLY,

//   based on WHERE we are, not the clock

.andThen(

new DriveToPose(drive, () -> targetPose) //  precise closed-loop to the exact pose

.until(() -> withinTolerance(...)));  //  ends on POSITION, not time

And DriveToPose itself — the part you want — is a profiled PID to a pose that finishes only when both translation and rotation are inside tolerance:

// 6328 DriveToPose - tolerances (defaults)

driveTolerance = 0.01;                       // 1 cm

thetaTolerance = Units.degreesToRadians(1.0);// 1 degree

public boolean withinTolerance(double dTol, Rotation2d thetaTol) {

return running

&& Math.abs(driveErrorAbs) < dTol

&& Math.abs(thetaErrorAbs) < thetaTol.getRadians();

}

// execute(): a TrapezoidProfile regenerated toward the target each loop,

//   PID on the distance-to-goal, with feedforward that fades out near the goal.

3467 (Windham Windup, the NE team that went 51–0) ships essentially the same tool as a reusable command with caller-set tolerances:

// 3467 DriveToPose - tolerance is a builder option

new DriveToPose(drive, () -> targetPose)

.withTolerance(Centimeters.of(2), Degrees.of(1));   // ends when BOTH are met

// isFinished() returns true only once linear AND angular error are within tolerance

The principle:  a time-based trajectory gets you close quickly and smoothly; a position-based DriveToPose guarantees arrival. Use the first for 90% of the travel, the second for the final approach. Never ask the timed path to be the thing that finishes a precision move.

### Three patterns top teams use to guarantee arrival

The handoff above is 6328’s version. Two other cloned teams solve the same problem differently:

| Team | Pattern | How the move ENDS |
| --- | --- | --- |
| 6328 | Trajectory → DriveToPose handoff | Timed path interrupted early on a spatial condition; DriveToPose finishes at 1 cm / 1° |
| 1768 Nashoba | Position-and-settle-gated trajectory | Completes only when translation AND rotation are within tolerance, HELD for a settle time; the nominal trajectory time is just a safety timeout |
| 3467 Windham | Reusable DriveToPose.withTolerance(dist, angle) | Ends when both linear and angular error are within caller-set tolerance |

Worth stealing first:  1768’s settle-timer. Requiring the robot to stay inside tolerance for a few hundred milliseconds before the move is declared done eliminates the “finished while still drifting” failure — and it is the smallest change to what you already run, because you bake it into the trajectory command instead of adding a second command.

## 4.3 Choreo vs. PathPlanner — which generator (and the alliance-flip myth)

Correcting a 2026 assumption:  the team avoided Choreo believing its paths could not be auto-reversed for red vs. blue. That is not true. ChoreoLib has built-in alliance flipping — Choreo.createAutoFactory(…, BooleanSupplier useAllianceFlipping, Supplier<Optional<Alliance>>), and sampleAt(t, flipped) / getFinalPose(flipped) take a flip flag. It supports both rotate-around (rotationally symmetric fields) and mirror (reflect across the centerline) flipping. Both 6328 and 1768 auto-flip their Choreo paths in the cloned code via AllianceFlipUtil.shouldFlip(). There is no alliance-flip blocker.

The real trade-off:

|  | Choreo | PathPlanner |
| --- | --- | --- |
| Optimizer | Time-OPTIMAL, dynamically feasible | Respects swerve dynamics, but not time-optimal |
| Tuning | “Follows from the start,” little tuning needed | More manual control; you can draw paths the robot can’t follow → more tweaking |
| Auto workflow | Trigger-based factory | Pre-made path-following commands / GUI |
| Ending | On time | On time |

For 2026 specifically: the community read is that this year’s paths are mostly straight-line drives, so the two generate near-equivalent results — not a game-changer this season; use what you know. They also interoperate: PathPlannerLib can follow Choreo .traj files and use them inside GUI autos, so it is not either/or.

- On precision: the generator choice is orthogonal to your arrival problem — both follow on time. Adopt Choreo for smoother/faster optimal paths and lower tuning if you like, but you still layer the settle-gate (1768) or DriveToPose end-controller on top.

- Final lineup: for the last centimeter-level alignment, pre-plan a short final segment or use a dedicated drive-to-pose PID — PathPlanner’s pathfindToPose() is for getting to the start of that segment, not for the precise finish.

# 5. Concrete plan for MechaRAMS

## 5.1 Summer prototype (on the 2025 chassis testbed)

| # | Task | Why / success check |
| --- | --- | --- |
| 1 | Implement a DriveToPose command on your stack (skeleton in 5.3). Wire it after a PathPlanner path with .until(atHandoffLine).andThen(driveToPose). | Robot ends within 2 cm / 1° of a taped target pose, 10/10 trials, regardless of approach speed. |
| 2 | Apply the prior-analysis global-pose fixes (MT2 yaw std-dev = ∞, fuse all cameras, dist² weighting). | Shrinks the baseline 2–3° heading error; measured against a known field pose. |
| 3 | Mount one global-shutter camera bore-sighted with the launcher; stream goal-tag bearing over NetworkTables. | Camera reports tag bearing whenever the goal tag is in frame; bearing repeatable to <0.5° at your shooting range. |
| 4 | Add a fine-aim loop: PID chassis (or turret) omega on measured tag bearing for the last ~3°; log poseBearing − cameraBearing. | Stationary shot heading settles to <0.5°; the logged residual is small and stable (your independent check). |
| 5 | Decide turret vs. fixed boresight from the data. | If the tag leaves frame during realistic aiming, you need a turret or a wider-FOV/forward-mounted cam; the log tells you. |

## 5.2 2027 architecture (the target)

- Two-tier localization. Global pose (multi-camera fusion on the roboRIO, 250 Hz odometry thread) for driving and coarse aim; a target-relative boresight measurement for the shot.

- Driving. PathPlanner or Choreo for transit; DriveToPose end-controller for every scoring/pickup/shoot pose, ending on position tolerance.

- Aiming. Global-pose coarse aim with velocity lookahead (6328 pattern) + boresight fine correction (the layer 6328 doesn’t have but you want, because you lacked the check).

- Observability. Log every shot’s pose-bearing vs. camera-bearing and every DriveToPose error trace, so “why did we miss in match 23” is a log lookup, not a guess.

## 5.3 Drive-to-pose skeleton (adapt to your subsystem)

A minimal, dependency-light version of the 6328/3467 pattern. Tune kP and tolerances on the testbed.

public class DriveToPose extends Command {

private final Drive drive;

private final Supplier<Pose2d> targetSupplier;

private final ProfiledPIDController xCtl, yCtl, thetaCtl;

private double distTol = 0.02, thetaTol = Math.toRadians(1.0); // 2 cm / 1 deg

public DriveToPose(Drive drive, Supplier<Pose2d> target) {

this.drive = drive; this.targetSupplier = target;

xCtl     = new ProfiledPIDController(3.0, 0, 0, new Constraints(3.0, 3.0));

yCtl     = new ProfiledPIDController(3.0, 0, 0, new Constraints(3.0, 3.0));

thetaCtl = new ProfiledPIDController(5.0, 0, 0, new Constraints(8.0, 8.0));

thetaCtl.enableContinuousInput(-Math.PI, Math.PI);

addRequirements(drive);

}

@Override public void execute() {

Pose2d pose = drive.getEstimatedPose();

Pose2d goal = targetSupplier.get();

double vx = xCtl.calculate(pose.getX(), goal.getX());

double vy = yCtl.calculate(pose.getY(), goal.getY());

double w  = thetaCtl.calculate(pose.getRotation().getRadians(),

goal.getRotation().getRadians());

drive.runVelocity(ChassisSpeeds.fromFieldRelativeSpeeds(vx, vy, w, pose.getRotation()));

}

@Override public boolean isFinished() {     // ENDS ON POSITION, NOT TIME

Pose2d pose = drive.getEstimatedPose(), goal = targetSupplier.get();

boolean atXY    = pose.getTranslation().getDistance(goal.getTranslation()) < distTol;

boolean atTheta = Math.abs(pose.getRotation().minus(goal.getRotation()).getRadians()) < thetaTol;

return atXY && atTheta;

}

@Override public void end(boolean i) { drive.stop(); }

}

// Usage - PathPlanner transit, then precise arrival:

AutoBuilder.followPath(transitPath)

.andThen(new DriveToPose(drive, () -> SCORING_POSE));

## 5.4 Boresight fine-aim skeleton

Closes the loop on the measured goal-tag bearing, and logs the independent check you were missing in 2026.

public class FineAimToGoal extends Command {

private final Drive drive; private final Vision boresight;

private final PIDController yaw = new PIDController(4.0, 0, 0.1);

private final double TOL = Math.toRadians(0.5);

@Override public void execute() {

// 1) Coarse heading from global pose (works even if tag briefly lost)

Rotation2d poseBearing = LaunchCalc.driveAngleFromPose(drive.getEstimatedPose());

if (boresight.hasGoalTag()) {

double camBearing = boresight.goalTagBearingRad();   // tag-relative, drift-proof

// 2) THE CHECK: pose-derived vs measured. Persistent gap = calibration error.

Logger.recordOutput("Aim/PoseVsCamBearingErr",

poseBearing.getRadians() - camBearing);

// 3) Fine correction closes on the MEASURED bearing

drive.runVelocity(new ChassisSpeeds(0, 0, yaw.calculate(camBearing, 0.0)));

} else {

// fall back to coarse pose aim

drive.runVelocity(new ChassisSpeeds(0, 0,

yaw.calculate(drive.getHeading().getRadians(), poseBearing.getRadians())));

}

}

@Override public boolean isFinished() {

return boresight.hasGoalTag() && Math.abs(boresight.goalTagBearingRad()) < TOL;

}

}

## 5.5 Decision summary

| Question you asked | Answer | What to do |
| --- | --- | --- |
| Do teams localize locally and globally separately? | Conceptually yes (6328’s framework); 6328 itself does aim from global pose with no turret, but logs/replays to keep it honest. | Adopt the two-tier framing; add the verification layer they have in logs and you didn’t have at all. |
| Should we add a turret/shooter camera? | Yes — as a fine-correction + check, not as the only source. It directly attacks the 2–3° angular error and gives the missing check. | Boresight a global-shutter cam to the launcher this summer; decide turret vs. fixed from frame-loss data. |
| Does NorthStar / custom processing fix driving precision? | No — that’s image processing. Even 6328’s follower ends on time. Precision comes from the end-controller, not the vision pipeline. | Keep PhotonVision for images; add DriveToPose for arrival. |
| Which odometry / how to update? | Keep CTRE as the 250 Hz sampler; feed a roboRIO estimator; apply vision at capture timestamp; never reset enabled (from prior analysis). | Carry forward the prior-thread fusion plan; layer DriveToPose + boresight on top. |
| Should we switch to Choreo? | Optional. The alliance-flip concern was a myth — ChoreoLib auto-flips. It is orthogonal to precision (still ends on time). In 2026 paths are mostly straight, so it’s near-equivalent. | Evaluate Choreo for smoother transit; keep the settle-gate / DriveToPose end-controller regardless of generator. |

# 6. Sources

- 6328 RobotCode2026Public — DriveTrajectory.java (time-based ending)

- 6328 RobotCode2026Public — DriveToPose.java (position-tolerance ending)

- 6328 RobotCode2026Public — LaunchCalculator.java (global-pose aim + lookahead)

- 3467 Skip-5.16-Perry — DriveToPose.java (tolerance builder)

- 6328 “Beyond the Coprocessor: Lessons in FRC Vision & Localization” (talk)

- 2910 Jack in the Bot — 2024CompetitionRobot-Public (turret aim: vision-direct + pose fallback)

- 1768 Nashoba — 2026 robot code (Choreo + cmdWithAccuracy settle-gated trajectory)

- 1678 Citrus Circuits — C2024-Public (separate GoalTracker + global pose)

- Chief Delphi — Vision-Odometry vs Direct AprilTag Alignment

- Chief Delphi — PathPlanner v. Choreo

- PathPlanner docs — Choreo interop

- Choreo docs — Trajectory API & alliance/sample flipping

- PathPlanner docs — pathfinding & final-lineup guidance